EM34 -> 6E5C

Usually both screens of the EM34 are used parallel. Therefore you´ll find two resistors mounted from PIN5 (L)to PIN3 (a1, d1) and from the same PIN5 (L) to PIN6 (a2, d2).

6E5C has only one screen hence only one resistor ( 1 MegOhm) will be used:

One Resistor is mounted from PIN6 (L) to PIN3 (A).

PIN6 (L) gets the wire which was originally mounted to PIN5 (L).

PIN5 (G) gets the wire which was originally mounted to PIN4 (G).

The socket has to be turned by about 45 degrees since the 6E5C Screen is aligned different to the EM34 ones.

We recommend keeping a sketch of the original wiring with the set.

Pinning EM34:

| PIN | Ident. |
| --- | --- |
| 1 | not used |
| 2 | f |
| 3 | a1, d1 |
| 4 | g |
| 5 | l |
| 6 | a2, d2 |
| 7 | f |
| 8 | k, g |

Pinning 6E5C:

| PIN | Ident. |
| --- | --- |
| 1 | not used |
| 2 | F |
| 3 | A |
| 4 | not used |
| 5 | G |
| 6 | L |
| 7 | f |
| 8 | K |